PICO

- Microcontrôleur RP2040 conçu par Raspberry Pi au Royaume-Uni

- Processeur ARM Cortex M0+ à double cœur, horloge flexible fonctionnant jusqu’à 133 MHz

- 264 Ko de SRAM et 2 Mo de mémoire Flash embarquée

- Le module peut se souder directement sur une carte

- Prise en charge de l’hôte et du périphérique USB 1.1 (OTG)

- Mise en veille à puissance réduite et modes « dormants »

- Programmation par glisser-déposer en utilisant le stockage de masse via USB

- 26 broches GPIO multifonctions

- 2 × SPI, 2 × I2C, 2 × UART, 3 × 12-bit ADC, 16 × canaux PWM contrôlables

- Horloge et minuterie précises intégrées

- Capteur de température

- Bibliothèques accélérées en virgule flottante intégrées

- 8 × machines d’état à entrées/sorties programmables (PIO) pour le support de périphériques personnalisés

## General board control¶

The MicroPython REPL is accessed via the USB serial port. Tab-completion is useful to find out what methods an object has. Paste mode (ctrl-E) is useful to paste a large slab of Python code into the REPL.

The machine module:

import machine

machine.freq()          # get the current frequency of the CPU

machine.freq(240000000) # set the CPU frequency to 240 MHz

The rp2 module:

import rp2

## Delay and timing¶

Use the time module:

import time

time.sleep(1)           # sleep for 1 second

time.sleep_ms(500)      # sleep for 500 milliseconds

time.sleep_us(10)       # sleep for 10 microseconds

start = time.ticks_ms() # get millisecond counter

delta = time.ticks_diff(time.ticks_ms(), start) # compute time difference

## Timers¶

RP2040’s system timer peripheral provides a global microsecond timebase and generates interrupts for it. The software timer is available currently, and there are unlimited number of them (memory permitting). There is no need to specify the timer id (id=-1 is supported at the moment) as it will default to this.

Use the machine.Timer class:

from machine import Timer

tim = Timer(period=5000, mode=Timer.ONE_SHOT, callback=lambda t:print(1))

tim.init(period=2000, mode=Timer.PERIODIC, callback=lambda t:print(2))

## Pins and GPIO¶

Use the machine.Pin class:

from machine import Pin

p0 = Pin(0, Pin.OUT)    # create output pin on GPIO0

p0.on()                 # set pin to "on" (high) level

p0.off()                # set pin to "off" (low) level

p0.value(1)             # set pin to on/high

p2 = Pin(2, Pin.IN)     # create input pin on GPIO2

print(p2.value())       # get value, 0 or 1

p4 = Pin(4, Pin.IN, Pin.PULL_UP) # enable internal pull-up resistor

p5 = Pin(5, Pin.OUT, value=1) # set pin high on creation

## Programmable IO (PIO)¶

PIO is useful to build low-level IO interfaces from scratch. See the rp2 module for detailed explaination of the assembly instructions.

Example using PIO to blink an LED at 1Hz:

from machine import Pin

import rp2

@rp2.asm_pio(set_init=rp2.PIO.OUT_LOW)

def blink_1hz():

# Cycles: 1 + 7 + 32 * (30 + 1) = 1000

set(pins, 1)

set(x, 31)                  [6]

label("delay_high")

nop()                       [29]

jmp(x_dec, "delay_high")

# Cycles: 1 + 7 + 32 * (30 + 1) = 1000

set(pins, 0)

set(x, 31)                  [6]

label("delay_low")

nop()                       [29]

jmp(x_dec, "delay_low")

# Create and start a StateMachine with blink_1hz, outputting on Pin(25)

sm = rp2.StateMachine(0, blink_1hz, freq=2000, set_base=Pin(25))

sm.active(1)

## UART (serial bus)¶

There are two UARTs, UART0 and UART1. UART0 can be mapped to GPIO 0/1, 12/13 and 16/17, and UART1 to GPIO 4/5 and 8/9.

See machine.UART.

from machine import UART, Pin

uart1 = UART(1, baudrate=9600, tx=Pin(4), rx=Pin(5))

uart1.write('hello')  # write 5 bytes

uart1.read(5)         # read up to 5 bytes

Note

REPL over UART is disabled by default. You can see the Getting started with MicroPython on the RP2xxx for details on how to enable REPL over UART.

## PWM (pulse width modulation)¶

There are 8 independent channels each of which have 2 outputs making it 16 PWM channels in total which can be clocked from 7Hz to 125Mhz.

Use the machine.PWM class:

from machine import Pin, PWM

pwm0 = PWM(Pin(0))      # create PWM object from a pin

pwm0.freq()             # get current frequency

pwm0.freq(1000)         # set frequency

pwm0.duty_u16()         # get current duty cycle, range 0-65535

pwm0.duty_u16(200)      # set duty cycle, range 0-65535

pwm0.deinit()           # turn off PWM on the pin

## ADC (analog to digital conversion)¶

RP2040 has five ADC channels in total, four of which are 12-bit SAR based ADCs: GP26, GP27, GP28 and GP29. The input signal for ADC0, ADC1, ADC2 and ADC3 can be connected with GP26, GP27, GP28, GP29 respectively (On Pico board, GP29 is connected to VSYS). The standard ADC range is 0-3.3V. The fifth channel is connected to the in-built temperature sensor and can be used for measuring the temperature.

Use the machine.ADC class:

from machine import ADC, Pin

adc = ADC(Pin(26))     # create ADC object on ADC pin

adc.read_u16()         # read value, 0-65535 across voltage range 0.0v - 3.3v

## Software SPI bus¶

Software SPI (using bit-banging) works on all pins, and is accessed via the machine.SoftSPI class:

from machine import Pin, SoftSPI

# construct a SoftSPI bus on the given pins

# polarity is the idle state of SCK

# phase=0 means sample on the first edge of SCK, phase=1 means the second

spi = SoftSPI(baudrate=100_000, polarity=1, phase=0, sck=Pin(0), mosi=Pin(2), miso=Pin(4))

spi.init(baudrate=200000) # set the baudrate

spi.read(10)            # read 10 bytes on MISO

spi.read(10, 0xff)      # read 10 bytes while outputting 0xff on MOSI

buf = bytearray(50)     # create a buffer

spi.readinto(buf)       # read into the given buffer (reads 50 bytes in this case)

spi.readinto(buf, 0xff) # read into the given buffer and output 0xff on MOSI

spi.write(b'12345')     # write 5 bytes on MOSI

buf = bytearray(4)      # create a buffer

spi.write_readinto(b'1234', buf) # write to MOSI and read from MISO into the buffer

spi.write_readinto(buf, buf) # write buf to MOSI and read MISO back into buf

Warning

Currently all of sck, mosi and miso must be specified when initialising Software SPI.

## Hardware SPI bus¶

The RP2040 has 2 hardware SPI buses which is accessed via the machine.SPI class and has the same methods as software SPI above:

from machine import Pin, SPI

spi = SPI(1, 10_000_000)  # Default assignment: sck=Pin(10), mosi=Pin(11), miso=Pin(8)

spi = SPI(1, 10_000_000, sck=Pin(14), mosi=Pin(15), miso=Pin(12))

spi = SPI(0, baudrate=80_000_000, polarity=0, phase=0, bits=8, sck=Pin(6), mosi=Pin(7), miso=Pin(4))

## Software I2C bus¶

Software I2C (using bit-banging) works on all output-capable pins, and is accessed via the machine.SoftI2C class:

from machine import Pin, SoftI2C

i2c = SoftI2C(scl=Pin(5), sda=Pin(4), freq=100_000)

i2c.scan()              # scan for devices

i2c.readfrom(0x3a, 4)   # read 4 bytes from device with address 0x3a

i2c.writeto(0x3a, '12') # write '12' to device with address 0x3a

buf = bytearray(10)     # create a buffer with 10 bytes

i2c.writeto(0x3a, buf)  # write the given buffer to the peripheral

## Hardware I2C bus¶

The driver is accessed via the machine.I2C class and has the same methods as software I2C above:

from machine import Pin, I2C

i2c = I2C(0)   # default assignment: scl=Pin(9), sda=Pin(8)

i2c = I2C(1, scl=Pin(3), sda=Pin(2), freq=400_000)

## I2S bus¶

See machine.I2S.

from machine import I2S, Pin

i2s = I2S(0, sck=Pin(16), ws=Pin(17), sd=Pin(18), mode=I2S.TX, bits=16, format=I2S.STEREO, rate=44100, ibuf=40000) # create I2S object

i2s.write(buf)             # write buffer of audio samples to I2S device

i2s = I2S(1, sck=Pin(0), ws=Pin(1), sd=Pin(2), mode=I2S.RX, bits=16, format=I2S.MONO, rate=22050, ibuf=40000) # create I2S object

i2s.readinto(buf)          # fill buffer with audio samples from I2S device

The ws pin number must be one greater than the sck pin number.

The I2S class is currently available as a Technical Preview. During the preview period, feedback from users is encouraged. Based on this feedback, the I2S class API and implementation may be changed.

Two I2S buses are supported with id=0 and id=1.

## Real time clock (RTC)¶

See machine.RTC

from machine import RTC

rtc = RTC()

rtc.datetime((2017, 8, 23, 2, 12, 48, 0, 0)) # set a specific date and

# time, eg. 2017/8/23 1:12:48

rtc.datetime() # get date and time

## WDT (Watchdog timer)¶

The RP2040 has a watchdog which is a countdown timer that can restart parts of the chip if it reaches zero.

See machine.WDT.

from machine import WDT

# enable the WDT with a timeout of 5s (1s is the minimum)

wdt = WDT(timeout=5000)

wdt.feed()

## OneWire driver¶

The OneWire driver is implemented in software and works on all pins:

from machine import Pin

import onewire

ow = onewire.OneWire(Pin(12)) # create a OneWire bus on GPIO12

ow.scan()               # return a list of devices on the bus

ow.reset()              # reset the bus

ow.readbyte()           # read a byte

ow.writebyte(0x12)      # write a byte on the bus

ow.write('123')         # write bytes on the bus

ow.select_rom(b'12345678') # select a specific device by its ROM code

There is a specific driver for DS18S20 and DS18B20 devices:

import time, ds18x20

ds = ds18x20.DS18X20(ow)

roms = ds.scan()

ds.convert_temp()

time.sleep_ms(750)

for rom in roms:

print(ds.read_temp(rom))

Be sure to put a 4.7k pull-up resistor on the data line. Note that the convert_temp() method must be called each time you want to sample the temperature.

## NeoPixel and APA106 driver¶

Use the neopixel and apa106 modules:

from machine import Pin

from neopixel import NeoPixel

pin = Pin(0, Pin.OUT)   # set GPIO0 to output to drive NeoPixels

np = NeoPixel(pin, 8)   # create NeoPixel driver on GPIO0 for 8 pixels

np[0] = (255, 255, 255) # set the first pixel to white

np.write()              # write data to all pixels

r, g, b = np[0]         # get first pixel colour

The APA106 driver extends NeoPixel, but internally uses a different colour order:

from apa106 import APA106

ap = APA106(pin, 8)

r, g, b = ap[0]

APA102 (DotStar) uses a different driver as it has an additional clock pin.